# ZET-5 Full Data Dictionary

This document outlines all persisted data schemas (stored in localStorage) and active telemetry flow variables used in the ZET-5 smart energy tracking system.

### 1. Key: zet5_setup (Onboarding Setup Configuration)

| Field Name | Data Type | Constraint / Format | Description |
| --- | --- | --- | --- |
| tokenData.kwh | Float | > 0, Units: kWh | Total prepaid electricity units purchased. |
| tokenData.amountPaid | Float | > 0, Currency (USD/ZWL) | Cost of the prepaid token transaction. |
| tokenData.purchaseDate | String | ISO-8601 Timestamp | Date and time the prepaid token was purchased and activated. |
| durationGoal | Integer | > 0, Units: Days (default: 21) | Target number of days the prepaid token is expected to last. |
| sensorNames | Array [5] | Strings (Max 20 chars) | Custom user-defined names for the 5 current sensing channels. |
| calibrationStart | Integer | Epoch Milliseconds | Timestamp of the start of the 7-day baseline learning phase. |
| notifyThreshold | Float | Units: kWh (default: 50) | Prepaid balance level below which Smart Advice alerts are triggered. |

### 2. Key: zet5_prediction_model (RHS Predictive Model State)

| Field Name | Data Type | Constraint / Format | Description |
| --- | --- | --- | --- |
| hourlySignature | Array [24] | Float, Units: kWh | The trained expected active consumption profile for each hour of the day (00:00 - 23:00). |
| hourlyAccumulator | Array [24] | Float, Units: kWh | Accumulates active consumption readings during the current hour. |
| sampleCount | Array [24] | Integer | Ticker sample counter used to calculate hourly average upon hour boundary crossing. |
| lastModelUpdate | Integer | Epoch Milliseconds | Timestamp of the last model write/sample update. |

### 3. Key: zet5_daily_averages (Historical Daily Averages Database)

| Field Name | Data Type | Constraint / Format | Description |
| --- | --- | --- | --- |
| date | String | YYYY-MM-DD | Date of the historical entry (stores up to 90 days). |
| sensors | Array [5] | Float, Units: Amperes (A) | Average current draw recorded for each sensor channel over the entire 24h day. |
| count | Integer | > 0 | Total samples collected during the day (for calculating mathematical average). |

### 4. Key: zet5_profiles (Appliance Profile Attributes)

| Field Name | Data Type | Constraint / Format | Description |
| --- | --- | --- | --- |
| name | String | String (Max 30 chars) | Name of the appliance mapped to the sensor channel. |
| base | Float | Units: Amperes (A) | Baseline current draw learned during calibration. |
| variance | Float | >= 0 | Expected operating variance in current. |
| type | String | Enum: inductive, resistive | The load characteristics (determines applied power factors). |
| maxLoad | Float | Units: Watts (W) | Maximum expected power draw threshold. |

### 5. In-Memory State & Telemetry Flow Parameters

| Parameter Name | Data Type | System Source / Destination | Description |
| --- | --- | --- | --- |
| sensors | Array [5] | Process 1 -> Process 2 | Real-time current measurements in Amperes (A). |
| activePower | Array [5] | Process 1 -> Process 2 | Calculated real power in Watts (W) per channel (P = V_rms * I_rms * cos phi). |
| k (kappa) | Float | Process 2 -> Store / UI | Calibration scaling coefficient used to correct integration drift. |
| kwhRemaining | Float | Process 2 -> Process 3 | Current remaining prepaid units in kWh. |
| daysRemaining | Float | Process 2 -> UI | Projected token runway before depletion (in days). |
| relays | Array [8] | Process 3 -> UI / Hardware | Actuator line states (true = power ON/closed, false = shed/open). |
| atRisk | Boolean | Process 3 -> UI | Flag indicating if projected runway depletion date is prior to the user's goal date. |